# UUNATEK Documentation

Version: 1.0   |   Date: February 2026   |   Framework: .NET 10.0

# Section 1 — Using the Application Class Library (DLL)

The UUNATRK.Application.dll class library contains all the core services for controlling the serial G-code printer. It can be referenced as a DLL in any .NET 10 or later project.

## 1.1 Prerequisites

Your consuming project must have the following NuGet packages installed:

| Package | Minimum Version | Purpose |
| --- | --- | --- |
| System.IO.Ports | 10.0.0 | Serial port communication with the printer |
| Microsoft.Extensions.DependencyInjection | 10.0.0 | Service registration |

## 1.2 Adding the DLL Reference

- Obtain the published UUNATRK.Application.dll file

- Place it in a known folder within your project (e.g., a lib folder)

- Add a reference to the DLL in your project file, pointing to where you placed it

## 1.3 Registering Services

The library provides a single extension method called AddApplicationServices that registers all services into the dependency injection container. Call it on your service collection. This registers:

- PrinterService — registered as a Singleton for serial G-code printing

## 1.4 Available Services

### PrinterService (Serial G-code Printer)

This service controls a pen plotter/printer connected via a serial (COM) port. It handles the full print lifecycle: connecting, sending G-code, handshaking with the machine, and paper ejection.

| Method | Description |
| --- | --- |
| OpenPort(comPort, baudRate) | Opens the serial connection. Default: COM5 at 250,000 baud |
| GetStatus() | Returns current status: IsOpen, PortName, IsPrinting |
| Print(gcode) | Sends a list of G-code commands and prints one copy |
| BulkPrint(gcode, copies) | Prints multiple copies (1–100) sequentially |

| Property | Description |
| --- | --- |
| IsOpen | Whether the serial port is currently open |
| PortName | The name of the connected COM port |
| IsPrinting | Whether a print job is currently running |

### SvgConverter (SVG to G-code)

A static utility class that converts SVG files into G-code commands ready to be sent to the printer.

| Method | Description |
| --- | --- |
| ConvertToGCode(stream, printRequest) | Reads an SVG stream and returns a list of G-code strings. Applies position, scale, rotation, and axis inversion based on the PrintRequest settings. |

## 1.5 Models

### PrintRequest

Used when converting SVG to G-code and printing on the serial printer.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| Width | String | 210mm | Paper width (e.g., "210mm") |
| Height | String | 297mm | Paper height (e.g., "297mm") |
| XPosition | String | 50mm | Horizontal offset on the paper |
| YPosition | String | 50mm | Vertical offset on the paper |
| Scale | Integer | 1 | Scale factor for the drawing |
| Rotation | Integer | 0 | Rotation in degrees (0, 90, 180, 270) |
| InvertX | Boolean | false | Mirror horizontally |
| InvertY | Boolean | true | Mirror vertically (inverted by default) |

## 1.6 Paper Sizes

The library includes a PaperSizes helper and a Paper enum for standard paper dimensions:

| Paper Type | Width (mm) | Height (mm) |
| --- | --- | --- |
| A3 | 297 | 420 |
| A4 | 210 | 297 |
| A5 | 148 | 210 |
| A6 | 105 | 148 |
| B4 | 250 | 353 |
| B5 | 176 | 250 |
| Letter | 216 | 279 |
| Legal | 216 | 356 |
| Tabloid | 279 | 432 |
| Card 4x6 | 102 | 152 |
| Card 5x7 | 127 | 178 |

Envelope sizes (10, 9, C5, C6, DL) are also available.

## 1.7 Namespaces

| Namespace | Contents |
| --- | --- |
| UUNATRK.Application | AddApplicationServices extension method |
| UUNATRK.Application.Services.Printer | PrinterService, SvgConverter |
| UUNATRK.Application.Models | PrintRequest, PrintResponse, PrinterStatus |
| UUNATRK.Application.Enums | Paper enum, PaperSizes helper |

# Section 2 — Using the Windows Desktop App

## 2.1 What to Install Before Running

The Windows desktop app requires one piece of software to be installed on the target machine:

| Requirement | Details |
| --- | --- |
| .NET 10.0 Desktop Runtime | Download from https://dotnet.microsoft.com/download/dotnet/10.0 — choose the “.NET Desktop Runtime” installer (not the base runtime or ASP.NET runtime) |

Important: You must install the Desktop Runtime specifically. The base .NET runtime does not include Windows Forms support and the app will not launch without it.

How to verify it is installed: Open a Command Prompt or PowerShell and type dotnet --list-runtimes. Look for Microsoft.WindowsDesktop.App 10.0.x in the output.

## 2.2 Running the App

- Extract the published folder to any location on the PC

- Double-click UUNATEK.WindowsForm.exe to launch the application

## 2.3 Step-by-Step Usage

### Step 1 — Connect to the Printer

- In the Serial Connection bar at the top, enter the COM Port your printer is connected to (e.g., COM5)

- Enter the Baud Rate (the default value of 250000 is correct for most setups)

- Click Connect

- The Printer Status bar will update to show IsOpen: True when the connection is successful

Tip: To find your COM port, open Device Manager on Windows, expand Ports (COM & LPT), and look for your printer/plotter device.

### Step 2 — Load an SVG File

- Click Browse... in the Simulation panel

- Select an SVG file from your computer

- The file name appears next to the Browse button, and the simulation preview shows the drawing on a virtual paper

### Step 3 — Adjust Print Settings

Use the Print Settings panel on the right side to configure how the drawing will be placed on the paper:

| Setting | Description | Example |
| --- | --- | --- |
| Paper | Select the paper size from the dropdown | A4, A3, Letter, etc. |
| X Position | Horizontal offset from the left edge of the paper | 50mm |
| Y Position | Vertical offset from the top edge of the paper | 50mm |
| Scale | Scale factor for the drawing (1 = original size) | 1, 2, 3... |
| Rotation | Rotate the drawing in degrees | 0, 90, 180, 270 |
| Invert X | Mirror the drawing horizontally | Unchecked by default |
| Invert Y | Mirror the drawing vertically | Checked by default |

The simulation preview updates live as you change any setting. This lets you see exactly how the output will look on paper before printing.

### Step 4 — Print

- Click the Print button at the bottom of the settings panel

- The app converts the SVG to G-code and sends it to the printer

- A result message appears showing the outcome (e.g., “Done (142 cmds)”)

## 2.4 Troubleshooting

| Problem | Solution |
| --- | --- |
| App won’t start or shows a runtime error | Install the .NET 10.0 Desktop Runtime from the Microsoft website |
| “COM port not found” error | Open Device Manager and check the correct COM port name under Ports (COM & LPT) |
| “Access denied” on COM port | Close any other application using the same port (e.g., Arduino IDE, PuTTY, or another instance of this app) |
| Print button does nothing | Ensure the printer is connected (IsOpen: True) and an SVG file is loaded |
| SVG not showing in the simulation | Verify the file is a valid SVG containing path elements |
| Drawing appears mirrored or upside down | Toggle the Invert X / Invert Y checkboxes in Print Settings |